FID-10: No Connection to Unit 🔌

The recovery steps depend on the module affected. Here are the details:

📡 ABIL (FSP Subunit Lost / No Connection to Unit)

Applicable SW: 25R3, 26R1, 26R2

- Wait for automatic recovery resets (3 times)

- If ABIL not recovered after auto recovery:

  - Perform BBMOD power off/power on (with 1-hour waiting period)

  - Perform Web-EM site power reset

📡 AEHC (No Connection to Unit)

Applicable SW: 25R3, 26R1, 26R2 | Waiting Period: 120 Minutes

- Collect Pre and Post Snapshot

- Engage R&D ASAP (No recovery before engaging R&D)

- Wait up to 2 hours for self-recovery (No TSH required within waiting period)

- Check if RU unit is reachable/pinging, then collect RU logs

📡 AHLOB / AHFII (No Connection to Unit)

Applicable SW: 25R3, 26R1, 26R2 | Waiting Period: 45 Minutes

- Collect Pre and Post Snapshot

- Check Rx power at BB from the unit

- Try SFP procedure and wait minimum 20 minutes

- The unit should recover on its own in most cases

📡 AHFII, AHLOA, AHLOB, AHFIG (FID-10 at NR End)

- LTE module is Up and Running

- ➡️ Perform pssreboot HW from LTE end using SSH

📡 AHFII, AHLOA, AHLOB, AHFIG (FID-10 at LTE End)

- NR module is Up and Running

- ➡️ Perform Web-EM RMOD reset from NR

⚠️ NOTE: During troubleshooting, if FID-10 turns into FID-1850 / FID-23 / FID-2934 / FID-2935 / FID-2942, it means connectivity to RU is established. Collect RU logs and snapshot at this point for R&D investigation.